Tailscale OLT Remote Access

Setup Guide for Rico Net ISP

Project: Remote OLT Management via Tailscale Subnet Routing

# 1. Overview & Network Architecture

This guide documents the complete setup for remotely accessing the OLT (Optical Line Terminal) device at the Rico Net ISP office from a Home PC, using Tailscale as a secure VPN tunnel with subnet routing.

Network layout:

- Home PC (amd) — 100.96.243.32 — Backend files live here

- Office Laptop (laptop-p2m9rcp8) — 100.73.137.94 — Connected via Wi-Fi + LAN cable to OLT

- OLT Device — 192.168.8.100 — Connected to Office Laptop via LAN cable on subnet 192.168.8.0/24

⚠ The OLT cannot run Tailscale itself — the Office Laptop acts as a subnet router, bridging the Tailscale network to the OLT's LAN.

# 2. Prerequisites

- Windows PC (Home) and Windows Laptop (Office) — both running Windows 11

- A single Tailscale account (free Personal plan is sufficient)

- OLT device physically connected to Office Laptop via LAN cable

- Office Laptop connected to internet via Wi-Fi

# 3. Step-by-Step Setup

## Step 1 — Install Tailscale on Both Machines

- Download Tailscale from https://tailscale.com/download

- Install on both the Home PC and the Office Laptop

- Sign in to the SAME Tailscale account on both machines

✅ Both machines will appear as Connected in the Tailscale admin panel at login.tailscale.com/admin/machines

## Step 2 — Advertise the OLT Subnet (Office Laptop)

On the Office Laptop, open Command Prompt as Administrator and run:

tailscale up --advertise-routes=192.168.8.0/24

This tells Tailscale to route any traffic destined for 192.168.8.x through this laptop.

⚠ Replace 192.168.8.0/24 with the actual subnet of your OLT if it differs. Check by running ipconfig and looking at the Ethernet adapter IP.

## Step 3 — Enable IP Forwarding on Office Laptop

Windows does not forward packets between network interfaces by default. Enable it by running this in Command Prompt as Administrator on the Office Laptop:

reg add HKLM\SYSTEM\CurrentControlSet\Services\Tcpip\Parameters /v IPEnableRouter /t REG_DWORD /d 1 /f

Then restart the Office Laptop for the change to take effect.

⚠ This step is critical. Without IP forwarding, the laptop receives packets from Tailscale but does not forward them to the OLT — causing timeouts.

## Step 4 — Approve the Subnet in Tailscale Admin Panel

- Go to: https://login.tailscale.com/admin/machines

- Find the Office Laptop in the list

- Click the three dots (...) next to it → Edit route settings

- Enable the checkbox next to 192.168.8.0/24 → Save

✅ After approval, the Office Laptop will show a 'Subnets' badge in the machines list.

## Step 5 — Accept Routes on Home PC

On the Home PC, open Command Prompt as Administrator and run:

tailscale up --reset

Then immediately run:

tailscale up --accept-routes

⚠ The --reset step is required if Tailscale shows an error about non-default flags. It clears all previous settings so you can apply --accept-routes cleanly.

## Step 6 — Verify the Connection

From the Home PC, test the connection by pinging the OLT:

ping 192.168.8.100

Expected result: 4 replies with 0% packet loss (latency ~100-200ms is normal over Tailscale).

Then open your browser and go to:

https://192.168.8.100:1010/action/main.html

✅ The OLT web management interface should load directly on your Home PC, just as it does from the office.

# 4. Final Setup Status

| Step | Status |
| --- | --- |
| Tailscale installed on both machines | ✅ Done |
| Both devices logged into same account | ✅ Done |
| Subnet 192.168.8.0/24 advertised on Office Laptop | ✅ Done |
| Subnet approved in Tailscale admin panel | ✅ Done |
| IP forwarding enabled on Office Laptop (registry) | ✅ Done |
| Home PC accepted routes (tailscale up --reset + --accept-routes) | ✅ Done |
| Ping to 192.168.8.100 from Home PC | ✅ Working |
| OLT web interface accessible from Home PC | ✅ Working |

# 5. Tailscale Free Plan — Limits & Pricing

Your current setup uses only 2 devices and 1 user, which fits entirely within the free Personal plan. No payment or upgrade is required.

| Plan | Price | Users | Devices |
| --- | --- | --- | --- |
| Personal (FREE) | $0/month | 3 users | 100 devices |
| Personal Plus | $5/month | 6 users | 100 devices |
| Starter (Business) | $6/user/month | Unlimited | 100+ devices |
| Premium (Business) | $18/user/month | Unlimited | 100+ devices |

## What the Free Personal Plan Includes

- Up to 3 users — you only need 1

- Up to 100 devices — you only need 2 (Home PC + Office Laptop)

- Unlimited subnet routers — no limit on the 192.168.8.0/24 routing you set up

- WireGuard encryption — all traffic is end-to-end encrypted

- No time limit — free forever, no credit card required

⚠ Important: The free Personal plan is intended for personal/homelab use. If this is used commercially by Rico Net as a business, you technically should use a paid Business plan (Starter at $6/user/month). For 1 user that would be $6/month.

# 6. Important Maintenance Notes

## Office Laptop Must Stay On

The Office Laptop is the bridge between Tailscale and the OLT. If it goes to sleep, disconnects from Wi-Fi, or the LAN cable is unplugged, remote access to the OLT will stop.

To prevent sleep, on the Office Laptop go to:

Settings → System → Power & Sleep → Sleep → Set to Never

## Key Expiry Warning

Tailscale's admin panel shows a 'Key expiry is enabled' warning on the Office Laptop. When the key expires, the Tailscale tunnel will be interrupted until you re-authenticate.

To disable key expiry (recommended for this setup), in the Tailscale admin panel click the three dots next to the Office Laptop → Disable key expiry.

## After Restarting the Office Laptop

After any restart, Tailscale should reconnect automatically. If subnet routing stops working, run this again on the Office Laptop as Administrator:

tailscale up --advertise-routes=192.168.8.0/24

# 7. Troubleshooting

## Ping times out from Home PC

- Check that the Office Laptop is on and connected to both Wi-Fi and LAN

- Re-run on Home PC: tailscale up --reset then tailscale up --accept-routes

- Re-run on Office Laptop: tailscale up --advertise-routes=192.168.8.0/24

- Verify the subnet is still approved in the Tailscale admin panel

## tailscale up --accept-routes gives 'non-default flags' error

This happens when Tailscale already has non-default settings. Fix it by running:

tailscale up --reset

tailscale up --accept-routes

## OLT web interface loads on office laptop but not Home PC

The routing is working (ping succeeds) but the browser is blocking it. Try:

- Use http:// instead of https:// in the URL

- If browser shows certificate warning, click Advanced → Proceed anyway

Document prepared for Rico Net ISP — Tailscale OLT Remote Access Project